Comments on Proposed FR-DE-NL Metric for QC Export Controls

The US greatly appreciates your efforts to develop potential alternatives for export controls on quantum computing.  Thanks greatly for sharing your thoughts with us on the national security issues and the proposed metric. US export control, national security, and quantum computing experts analyzed the potential metric you proposed and identified several issues/questions with the proposed approach that we want to discuss with you.

This paper is organized in two sections.  First, we offer some general comments on the proposed language, mainly from the export control point of view.  The second section offers a detailed analysis of the slides you presented during our experts meeting.

General Comments:

Proposed metric:

“Quantum computers and related ‘electronic assemblies’ and components specially designed to support a ‘universal set of gates’ between error corrected qubits showing errors smaller than ‘physical errors’ by a factor of 10.”

Although the actual performance of Quantum Error Correction (QEC) is greatly affected by the specific gate and qubit hardware implementation (which could be designed to facilitate the implementation of a specific error correction scheme), the proposed metric depends predominantly on the effectiveness of the QEC software.  That is, error correction schemes can be implemented by software and the results of the same QEC algorithm will vary significantly among different hardware implementations (e.g., different superconducting-qubit-based QC systems).  Furthermore, it is possible for the software to be developed (and implemented) by a third party (it could be open source) not necessarily the hardware vendor.  Although companies working on large-scale quantum computing R&D also work on QEC, the preponderance of this work is, today, happening in academia.  In fact, large companies such as Microsoft, Google, IBM, and many others rely on a vast network of university partners to do most of their QEC research.  Thus, this proposal will impact smaller R&D systems to a higher degree than just focusing on industrial-scale quantum computing hardware.  Of course, the hardware could also be sold without the software, to be later retrofitted by the customer with commercial or indigenous QEC software, thereby circumventing the control language completely.

It might be instructive to look at a hypothetical case based on actual hardware and R&D practices of a company; in this case IBM.  Currently IBM’s 127-qubit system has much lower two-qubit gate fidelities than the 27-qubit Falcon r10 system.  The reason for this is that IBM uses the smaller system to research gate improvements and error correction schemes while using the large systems to research and further scalability schemes.  So, we can conceive of a scenario where a factor of ten error correction improvement is achieved in the smaller system. In that case the smaller 27- qubit system would be controlled but not much larger systems such as the 127-, 1000-, or even the over-4000-qubit (Kookaburra) systems.

A second scenario is based on the fact that logical qubits and gates have been demonstrated on small ion trap systems.  Such demonstrations have been done on relatively small systems (tens of qubits) which do not seem, at this moment, easily scalable to hundreds of qubits or beyond.  One of those demonstrations was done using a university QC.  So again, there is the possibility that the proposed language could control these less capable systems while more scalable systems from industry would not be controlled.

Comments on Revised FR 004 Proposal Slides

Slide 2: “US concern: The control should be based on a relevant breaking point from which the uncertainties on the development of scalable quantum computers become low enough.”

The rationale underlying the US proposal is somewhat different than stated above.  US 008 is intended to deny access to technological innovations in the form of logical and physical architectures, advanced components, and modes of integration and operation (e.g., scalable techniques for thermal management, noise and cross-talk suppression, and efficient signal bandwidth management), and to thereby prolong adversarial system development timelines to the maximum degree possible.  The US believes this policy will have maximum impact in delaying adversarial initial operating capability (IOC) if enforced from the very earliest stages of such development programs, long before major risks and uncertainties have been significantly reduced.  As such, the “breaking point” for the US is the transition from the non-scalable, academic-style laboratory technology used up until very recently to the much more advanced and scalable technology now being progressively incorporated by commercial and industrial developers.  This transition in technology base is not necessarily tied to the quantum computational performance of the successively larger processors used to mark the milestones in the various development roadmaps, but it does closely track the increase in their scale, i.e., in their physical qubit (PQB) count.  Peter Shadbolt, the Chief Strategy Officer of PsiQuantum explains it thusly:

“We have a whole chain of incrementally larger and larger systems that we’re building along the way. Those allow us to validate the control electronics, the systems integration, the cryogenics, the networking, etc. But we’re not spending time and energy, trying to dress those up as something that they’re not. We’re not having to take those things and try to desperately extract computational value from something that doesn’t have any computational value. We’re able to use those intermediate systems for our own learning and for our own development.”

Slide 3: “Hundreds of error corrected qubits are needed to run production codes in the above listed applications. Such machines are not expected to be available, even in labs, before at least a decade.  French and other European security agencies expect that large companies will have migrated their critical information systems to quantum safe standards in a timeframe of ten years. Quantum safe algorithms are expected to become available within Internet’s Transport Layer Security (TLS) protocol in market leading web browsers and in major cryptographic libraries in the next five years.”

The above could be construed to imply that the threat is relatively minor and short-lived.  The US believes that its National Security Systems will not be fully secured before 2035 and that the private sector will likely take even longer.  In addition, the threat is not only to “information in motion,” but also to “information at rest.”  Encrypted data dealing with industrial secrets, defense system vulnerabilities, intelligence operations, etc. frequently remains sensitive for decades after it has been generated and, in some cases, indefinitely (e.g., nuclear weapons design data).  Such data can be collected and archived for later decryption when the required computational cryptanalytic technology becomes available (so-called back-traffic attack).  The amount and sensitivity of data that remains vulnerable to such back-traffic attack even after deployment of cryptographic countermeasures tends to decrease over time, so the greater the delay in an adversarial IOC, the less damage it can inflict, but some level of damage can be expected for decades following deployment of post-quantum cryptography.

Slide 5: “Quantum computers and related ‘electronic assemblies’ and components specially designed to support a ‘universal set of gates’ between error corrected qubits showing errors smaller than ‘physical errors’ by a factor of 10.”

Without endorsing this proposal, the US suggests the following wording changes: “Quantum computers and related ‘electronic assemblies’ and components specially designed to support a ‘universal set of gates’ between error corrected qubits showing ‘logical errors’ smaller than ‘physical errors’ by a factor of at least 10.”  (Error corrected qubits have both physical and logical errors.)

The IBM Quantum Roadmap shows that quantum error correction (QEC) will not be implemented on their processors until they reach at least the 10,000-PQB level, whereas the IonQ Quantum Roadmap shows them beginning to implement QEC on their 64-PQB processor, which will support 4 logical qubits (LQBs).  Similarly, Rigetti plans to incorporate QEC at 4,000+ PQBs, while Google plans to begin doing so at 100 PQBs.  So, the impact of the revised FR 004 language is likely to be wildly disparate, depending on the particular development strategy being pursued.  It’s, of course, likely that, e.g., IBM will be experimenting with the implementation of QEC on some of their smaller processors (an activity that doesn’t appear on their public roadmaps).  Although such error-corrected processors would be controlled to the extent that they meet the control criteria of the revised FR 004 language above, all of IBM’s larger processors containing the advanced engineering and technology developments being incorporated to eventually support large-scale, fault-tolerant QEC would remain uncontrolled through, and beyond, their 4,158-PQB Kookaburra processor.

It may thus be relatively easy to circumvent the proposed control language.  Companies producing error-corrected processors that would be controlled by the revised FR 004 criteria could simply offer an “export version” of their processor without the QEC software.  If everything else was included (e.g., hardware to reduce latency in syndrome extraction and feedback), customers could conceivably devise and run their own QEC software.  Companies could also offer an export version with lower performance qubits (e.g., qubits employing lower purity materials or sub-optimal designs) in order to bring the ratio of logical-to-physical gate errors up sufficiently to “pass” the revised FR 004 criteria for export.  (Note that

pLogicalpPhysical=pPhysicalpThresholdd-12,                                                                                       (1)

where pLogical and pPhysical are the average logical and physical gate error probabilities,  pThreshold is the gate error probability threshold of the code, typically 10-2 for a surface code, which is fixed, and d is the distance of the code, which would generally also be fixed in most instantiations.  Because of the exponent on the right-hand side of Eq. (1), a relatively small change in PQB performance will be strongly amplified in its effect on pLogical/pPhysical.  For example, suppose pPhysical/pThreshold=10-1 and d=7, which requires about 100 PQBs/LQB, then pLogical/pPhysical=10-3≪10-1.  If pPhysical/pThreshold is increased to 5 x 10-1, i.e., pPhysical is increased from 10-3 to 5 x 10-3, assuming pThreshold is 10-2, pLogical/pPhysical increases from 10-3 << 10-1 to 0.125 > 10-1.  This increase in average physical gate error could thereby give adversaries unregulated access to an otherwise export-controlled, high-precision control system, which they could potentially reverse engineer and use together with their own higher fidelity qubits.)

Slide 6: “FR/DE consider that the number of qubits would be a metric not sufficiently descriptive, appropriate, universal, robust and sustainable, to mitigate the proliferation issues related to quantum computing.”

Because the number of error-free gates that can be performed with an average physical gate error probability of pPhysical is pPhysical-1, the number of time-steps that can be executed before incurring a fatal error is N-1pPhysical-1, where N is the number of PQBs and we are assuming that every PQB is involved in some type of gate, including idling gates, during every time-step.  Even assuming an average physical gate error probability that is an order of magnitude better (less) than the current SOA average physical gate error probability of 3 x 10-3, would allow for only about 15 time-steps with 200 PQBs and progressively fewer with increasing PQB count.  It, therefore, makes no sense to continue to scale to larger numbers of PQBs – even for NISQ systems, which are not attempting to realize QEC – if one is not simultaneously incorporating increasingly sophisticated and precise control technology (one of the major factors in both improving qubit performance and creating the infrastructure to enable large-scale QEC).

So, in this particular historical circumstance, the seemingly simple and naïve metric of PQB count is an excellent proxy for what we wish to control, viz., the advanced technology development necessary to enable quantum computer scale-up.  In October 2019, Prof. Scott Aaronson, a major authority in the field, was asked by the New York Times to explain the then recent “quantum supremacy” demonstration by Google and its wider significance.  In his article, he wrote:

“… researchers think that quantum error correction is what will ultimately let quantum computers scale beyond a couple hundred qubits, to the million- or billion-qubit machines that would fully realize Feynman’s dream.”  [Emphasis added.]

So, while 200 PQBs isn’t a magic number, there does seem to be a general consensus that scaling up beyond that point requires simultaneous efforts to develop the scalable system designs and high-precision hardware (both classical and quantum) necessary to enable large-scale, fault-tolerant QEC.  It is the unreviewed transfer of that technology to adversaries that US 008 is attempting to prevent.

Summary of Comments and way ahead:

The proposed language could have wildly disparate impact, conceivably snagging a 256-PQB IonQ processor running a d = 7 surface code with an average physical gate error of 3 x 10-3, but not an IBM Kookaburra processor with 4,158 PQBs, lower gate error rate, and most of the technology to support much larger scale QEC.

For an otherwise controlled processor, the proposed language allows for easy circumvention of the control via 1) elimination of its QEC software; 2) use of lower performance qubits; or 3) use of a lower-distance or otherwise less-efficient code, thus permitting unreviewed export of a qubit control system that is nevertheless capable of meeting or exceeding the export control criteria.

We could remedy the above shortcomings by modifying the proposed language to include platforms with average physical gate error and PQB count that is potentially sufficient to realize a pair of LQBs with logical-to-physical gate error ratio ≤ 10-1, but for which QEC may not be implemented, or not be implemented optimally.  However, this would entail a lot of analysis (beyond what a licensing officer could do).  The actual, as opposed to ideal, performance of QEC codes, e.g., depends on many implementation-specific factors, including qubit layout, communications latency, and the nature of the physical errors.

We are discussing the way ahead on US 008 internally.  This conversation includes an exploration of potential modifications inspired by your suggestion on using gate error rates as a metric, although we still believe that the PQB number is better aligned with our national security interests.  We hope to have a joint way forward before the next meeting.  Therefore, we want to continue the conversation between experts and want to suggest that we get together (virtually) again the first week of September if possible.   Meanwhile please let us know your comments on these remarks and any suggestion for a way forward.